# Telephony Requirements - ISP Sample

AI Contact Center - Phase 1 ISP Developer Test Documentation

HimalNet Broadband Pvt. Ltd.

Synthetic ISP Sample - Developer Test Data Only. All customer names, phone numbers, IP addresses, URLs, credentials, prices, network elements and records are fictional and must not be used in production.

# 1. SIP Provider Details

| Parameter | Synthetic Phase 1 Value |
| --- | --- |
| SIP Provider | TestTel SIP Lab |
| Trunk Type | SIP trunk over TLS preferred; UDP allowed only in isolated test lab |
| SBC / Proxy | sip-test.provider.example |
| Signaling Port | 5061 TLS / 5060 UDP test |
| Media | RTP/SRTP depending on lab capability |
| Codecs | G.711 A-law / μ-law; Opus for WebRTC/internal where supported |
| DTMF | RFC 2833 / RFC 4733 |
| Concurrent Calls | 20 test channels |
| Caller ID | Synthetic test DIDs only |
| SIP Authentication | Secret stored in vault; never embedded in prompts or documents in production |

# 2. Existing PBX Information

| Item | Sample |
| --- | --- |
| PBX | Asterisk 22 LTS test environment |
| AI Entry Extension | 7000 |
| Customer Care Queue | 8001 |
| NOC L1 Queue | 8002 |
| Billing Queue | 8003 |
| Sales/Retention Queue | 8004 |
| Supervisor Queue | 8090 |
| Recording | Enabled in test environment according to configured policy |
| Call ID | PBX uniqueid mapped to AI conversation_id and CRM call_id |

Recommended media/control flow: SIP Provider -> SBC/Asterisk -> AI Voice Gateway -> streaming STT -> Orchestrator/LLM + RAG/Tools -> streaming TTS -> Asterisk -> Caller. Human handoff returns the live call to an Asterisk queue while the integration layer pushes structured context to the agent desktop/CRM.

# 3. IVR Flow

Play approved call recording / AI-assistant disclosure.

Language: “Press 1 for English, 2 for Nepali, or speak naturally.” Automatic detection may bypass the menu when confidence is high.

Ask open intent question: “How can I help with your internet service today?”

Classify into Technical / Billing & Renewal / Package & Sales / Installation & Relocation / Complaint / Human Agent.

For Technical, ask for service identifier and verify only when account-specific data is needed.

AI performs ISP diagnostic workflow with relevant tools.

If unresolved or escalation trigger fires, select queue based on fault domain and priority.

Before transfer, play a short transition and send context to the agent desktop/CRM.

If human queue is closed, offer approved callback/ticket path; do not falsely claim a live agent is available.

## 3.1 DTMF Fallback Menu

| Key | Route |
| --- | --- |
| 1 | Technical Support |
| 2 | Billing / Renewal / Payment |
| 3 | New Connection / Package / Upgrade |
| 4 | Installation / Relocation |
| 0 | Human Agent / Operator |
| 9 | Repeat menu |

# 4. Phone Numbers

| Purpose | Synthetic Number / Identifier |
| --- | --- |
| Main Inbound Support | +977-01-555-0100 (TEST ONLY) |
| Toll-Free Test DID | 1660-TEST-ISP (placeholder) |
| Outbound CLI | +977-01-555-0199 (TEST ONLY) |
| Internal AI Extension | 7000 |
| Customer Care | 8001 |
| NOC L1 | 8002 |
| Billing | 8003 |
| Sales/Retention | 8004 |

# 5. Call Routing Rules

| Condition | Primary Route | Fallback / Notes |
| --- | --- | --- |
| No internet + LOS/ONU optical alarm | NOC-L1 / FIELD-OPS | If active shared incident, link outage before field dispatch. |
| No internet + account suspended/expired | BILLING/self-service | Avoid technical dispatch unless independent network fault exists. |
| No internet + no AAA session, billing active | NOC-L1 | Include auth/provisioning reason codes. |
| Slow speed Wi-Fi only | CARE-L1 / AI self-resolution | Transfer to technical if unresolved. |
| Wired performance issue / congestion alarm | NOC-L1 | Escalate L2 for recurring/systemic issue. |
| Payment dispute | BILLING | Supervisor for high-risk dispute. |
| New connection / package inquiry | SALES | Create lead if integration available. |
| Cancellation intent | SALES-RETENTION | Human handling preferred. |
| Caller says agent/human/operator | Best matching human queue | Do not force continued AI conversation. |
| Security/account takeover concern | SECURITY / Supervisor | Stop sensitive actions. |

# 6. SIP / Voice Test Cases

| TC | Test | Pass Condition |
| --- | --- | --- |
| TEL-01 | Inbound G.711 call to AI extension | Two-way audio stable; call IDs correlated. |
| TEL-02 | Nepali speech recognition | Intent/keywords recognized sufficiently to execute correct flow; fallback offered on low confidence. |
| TEL-03 | Barge-in while TTS speaking | Playback stops and caller speech is processed without losing conversation state. |
| TEL-04 | DTMF fallback | Keys route to correct menu/intent. |
| TEL-05 | Human transfer | Live call reaches queue; agent receives summary and verification state. |
| TEL-06 | Queue unavailable | AI follows after-hours/callback policy; no dead air or false availability. |
| TEL-07 | Long silence/no input | Reprompt, then fallback/termination policy. |
| TEL-08 | API latency during call | AI uses short hold/filler prompts and does not fabricate result. |
| TEL-09 | SIP re-INVITE/hold transfer behavior | Media remains valid according to PBX design. |
| TEL-10 | Call recording metadata | Recording reference stored only when policy allows and linked to call ID. |